ITEM NO. 11 (B-28)

(1) Name of the work :	Construction of 11 KV Electric Substation at Cross Road No.2, Africa Avenue, Sarojini Nagar, New Delhi

(2) Name of the Department  :  Electricity

(3) Brief History of the case :

The load has increased around Sarojini Nagar and its surrounding colonies during the last few years. In summer & rainy season frequent breakdowns and interruptions are observed. It was  highlighted by the media and local residents also. It was, therefore, felt necessary that new sub-stations should be established in the area.

In this connection Chairperson NDMC had also written a letter dated 1-2-2001 to Minister of Urban Development. Govt. of India for allotment of sites for 220 KV, 66 KV, 33 KV & 11 KV voltage levels for installation of Electric Substations at various places in NDMC area.

A plot for establishing an Electric Substation having 2 x 1000 KVA transformers at Cross Road No.2, Africa Avenue, Sarojini Nagar, New Delhi  has been allotted by L&DO.  The drawing of the same has also been prepared by C.A., NDMC.  This proposed Sub-station would conceptually be a local distribution Sub-station as well as a switching station.

(4)  Detailed proposal on the Project :

The sub-station shall have provision of one no. H.T. Board having 12 nos. panels and following HT cables have been proposed :-

- 2 x 400 sq.mm 11 KV XLPE cable has been proposed from 33 KV Sub-station Netaji Nagar as incomer to S/S Cross Road NO.2.

- 1 x 400 sq.mm 11 KV XLPE cable has been proposed from Sub-station Trauma Centre to I- Block.

- 1 x 400 sq.mm 11 KV XLPE shall be provided as outgoing from Proposed Sub- Station Cross Road No. 2 to S/S K-Block

- H.T feed of 1 x 400 sq.mm 11 KV  from S/S H-Block to S/S K-Block shall be taken out from S/S K- Block and extended up to 33 KV S/S NBCC.

- 2 nos. 1000 KVA oil type transformers shall be provided at S/S Cross Road No.2

One HT panel shall become spare at Electric Substation K-Block after the inter-connector between S/S H-Block and S/S K-Block is disconnected at S/S K-Block This spare HT panel at S/S K-Block  shall be utilized to connect HT feed 1 x 400 sq.mm 11 KV  XLPE coming from the proposed 11 KV feed from Electric Substation Cross Road No. 2 .Further, 2 x 1000 KVA oil type transformer shall be installed, out of the transformers which would be spare after replacement/ augmentation of oil type transformer with dry type transformer (Phase-II).  50% cost of new transformer as indicative cost has been taken in this estimate on account of providing old dismantled transformer. The service cable for IEI of sub- station have been proposed from near by G.S. Pillar.

(5)	Financial implications : Financial implications in the proposal is anticipated to be Rs. 2,28,14,000/- which includes the cost of  electrical work and civil work.

(6)	Implementation schedule: Six  months

(7)	Comments of the Finance Deptt. on the subject :  Finance Deptt. vide Dy. No. 4790/PA/Dir(F) dated  14-1-10  have concurred with no further comments.

(8)	Comments of the Deptt. on the Comments of the Finance  Deptt. : NIL

(9)	Legal implication of the Project : - No legal implication is involved.

(10)	Details of previous Council Resolution :	NIL

(11)	Comments of  Law Deptt. :  	NIL

(12)	Comments of the Deptt. on the comments of Law Deptt. :  NIL

(13)	Certification by the Deptt. that all Central Vigilance Commission (CVC) guidelines have been followed : It is certified that all Central Vigilance Commission (CVC) guidelines have been followed in processing the case.

(14)	Recommendation of the Deptt. : The estimate amounting to Rs.2,28,14,000/- for Construction of 11 KV Electric Substation at Cross Road No.2, Africa Avenue, Sarojini Nagar, New Delhi  may be placed before the Council to accord administrative approval and expenditure sanction.

(15)	Draft Resolution : Resolved by the Council that Administrative approval and expenditure sanction is granted to the estimate amounting to Rs.2,28,14,000/- for Construction of 11 KV Electric Substation at Cross Road No.2, Africa Avenue, Sarojini Nagar, New Delhi.

COUNCIL’S DECISION

Resolved by the Council to accord administrative approval and expenditure sanction to the estimate amounting to Rs.2,28,14,000/- for the Construction of 11 KV Electric Substation at Cross Road No.2, Africa Avenue, Sarojini Nagar, New Delhi.

It was also resolved that further action may be taken by the Department in anticipation of confirmation of the Minutes by the Council.