ITEM NO. 09 (B-31)

- NAME OF WORK:

Laying of 33KV, 3Cx400sq.mm (XLPE) cable emanating from 220/33KV Trauma Centre to various 33KV ESS.

- Name of the Department- ELECTRICITY DEPARTMENT

- Brief history of the proposals-

The Electric Sub-Station 220/33 KV at Trauma Centre with transformation capacity 200 MVA (2x100 MVA) is to be established by M/S DTL in order to provide system reliability and strengthening of power network. Due to saturation limit of existing Sub-stations 220/66KV  at Park Street, Ridge Valley (under commissioning), Sri Fort & Kirlokri grid and restricted evacuation of power demand from the DTL sub-stations, it has become very essentials to establish 220/33KV Sub-stations in the NDMC areas by the DTL to meet the growing demand of power supply. Further, this plan is also considered under the Functional Plan 2021 to meet the load growth pattern of power supply in NDMC area.

Accordingly, it has been proposed to provide various feeders of 3CX400 mm2 for utilization of transformation capacity of the 220/33KV ESS. These emanating feeders will add more reliability and strengthening to the existing and new 33/11KV ESS to provide uninterrupted power supply to the VVIP area.

- Detailed proposal on the Project-

For providing smooth and uninterrupted power supply from the existing 220KV ESS Trauma Centre following is being proposed: -

- Laying of 33KV, 400mm2/3C  XLPE Cable from 220KV ESS Trauma Centre to33KV ESS Trauma Centre (2 x .525 Km)……….....1.05 Km.

- Laying of 33KV, 400mm2/3C  XLPE Cable from 220KV ESS Trauma Centre to 33KV ESS AIIMS (3 x 2Km)…………...……6 Km.

- Laying of 33KV, 400mm2/3C  XLPE Cable from 220KV ESS Trauma Centre to 33KV ESS Kidwai Nagar (2 x 1.6Km).….3.2 Km.

- Laying of 33KV, 400mm2/3C  XLPE Cable from 220KV ESS Trauma Centre to 33KV  ESS Rajiv Gandhi Bhawan .…...0.525 Km.

- A feeder of size 3Cx400mm2 (XLPE) is already laying between 33KV ESS Trauma Centre and 33KV ESS Rajiv Gandhi Bhawan. The existing cable shall be dismantled from existing 33KV ESS Trauma Centre and shall be terminated to new 220/33KV ESS Trauma Centre for which additional cable of 525 meters is required.

- Laying of 33KV, 400mm2/3C  XLPE Cable from 220KV ESS Trauma Centre to 33KV  ESS State Guest House..………0.525 Km.

- A feeder of size 3Cx400mm2 (XLPE) is already laying between 33KV ESS Trauma Centre and 33KV ESS State Guest House. The existing cable shall be dismantled from existing 33KV ESS Trauma Centre and shall be terminated to new 220/33KV ESS Trauma Centre for which additional cable of 525 meters is required.

- Financial Implications-

An estimate amounting to Rs 248.37 Lacs including 3% contingencies has been prepared to cover the cost of the above work.

The expenditure shall be chargeable to the Head of A/c E-4-1 under the Original works for which a budget provision of Rs. 124.185 Lacs has been  proposed in the RE 2010-11 and subsequently in BE-2011-12.

- Implementation schedule with timeline for each stage including internal processing: -.

The scheme will be complete within 10 months after sanction of estimate &shall be executed departmentally / contract.

.

- Comments of the Finance Department on the subject:-

The Finance Deptt. has concurred the estimate vide diary No. 2375 dated 02.11.2010 with  no further comments.

- Comments of the Department on comments of Finance Deptt:-

No Comments

- Legal implications of the Project: -

No legal implication is involved.

- Details of Previous Council Resolution:-

Nil

- Comments of the Law Deptt. on the Project:

Nil

- Comments of the Deptt. on the Law Deptt:

Nil

- Certificate on CVC Guidelines:-

All CVC guidelines shall be adhered to while processing the case.

- Recommendation:-

The estimate amounting to estimate amounting to Rs 248.37 Lacs for Laying of 33KV, 3Cx400sq.mm (XLPE) cable emanating from 220/33KV Trauma Centre to various 33KV ESS may be placed before the Council to accord Administrative Approval and Expenditure Sanction.

- Draft Resolution:-

Resolved by the Council that Administrative Approval & Expenditure Sanction is granted to the estimate amounting to Rs 248.37 Lacs for Laying of 33KV, 3Cx400sq.mm (XLPE) cable emanating from 220/33KV Trauma Centre to various 33KV ESS.

COUNCIL’S DECISION